# Nucleo-based   Teaching Drone

# Project Report

Ellinger Matthias

### DONEs

- Writing IMU settings into registers

- Reading out raw IMU data

- Writing out PMW values to the motor drivers

- Calculating Roll and Pitch from accelerometer data

- Combining gyroscope values with accelerometer data by applying a Kalman filter

- PID control loop controlling motors pmw

- Interrupt routing decoding IR remote

- Remote controlled desired angle adjustment

### TODOs

- Saving orientation data onboard when disconnected for later interpretation

- Python application to automatically read out and graph saved sensor data

- PID self-tuning with Kalman filter as per https://doi.org/10.1016/j.ifacol.2018.06.131

### Implementation:

A flow chart of the program can be seen below. On startup the IMU gets configured and timers are started. In the main while loop flags are continuously being checked, these flags are set by interrupts. A 500 Hz timer interrupts the while loop to acquire new data, process it, apply the Kalman filter and finally use the PID function to output new PWM values to the motors. In a GPIO interrupt routine the IR Remote is being decoded by comparing the time between interrupts to threshold values and saving long impulses as 1s and short ones as 0s in bits of an integer. After the 32 impulses are sent the resulting 32-bit integer is compared to the know value of each button and the corresponding flags for starting, stopping or changing the orientation are set.